# Some basic terms in area of Telecommunications Networks:

- 4-Wire (4W) transport (switching) –  such transport (switching), which uses separated paths for each (1 wire pair for each)  from both directions (for backhaul) – in the central offices – see also [1] /p.28.

- Duplex – the simultaneous (contemporary) information transport in both directions

- 2-Wire (2W) transport (or switching)  - such simultaneous transport (switching) in both directions of backhaul, which uses single path for both directions (only 1 pair of wires) (e.g. the connecting of telephone to local exchange)

- 2W/4W hybrid – a circuit which connects (transforms) 2W local loop to 4W network

- Information  - the messages or knowledge, which can be expressed by the form suitable for communication, storage or for processing.

- Channel (transport ch.) – the complex of technical devices or tools, which allow the transport of signals in one direction between 2 points.

- Compressor   / expander - the switching equipment allowed the connections between loops of its one side and different number of loops of the other side (from input to output).

- Modulation – a process of varying (modifying) of some characteristic signal value by characteristic value of another signal.

- Multiplexor - a communications device that multiplexes (combines) several signals for transmission over a single medium.

- Multiplexing  - a process combining (channelizing) signals from several separated sources to unique complex signal dedicated to transport by common transmission channel (transmission line).

- Demultiplexor realizes the process of separating multiplexed signals from a transmission line. Frequently a multiplexor and demultiplexor are combined into a single device capable of processing both outgoing and incoming signals.

- Ring (Loop, Circuit) (telecommunication circuit) – 2 unidirectional channels allowed a transmission between 2 points.

- Transport path – path of transported signal between 2 points

- Switching – the process of building the request connection from one subscriber to another in the switching systems (exchanges)

- Circuit switching – interconnecting of end devices, transport channels or telecommunications circuits for the period of calling or service

- Packet switching -  process of packet messages routing (packets of messages with addresses, ..., packets storing, completing and decoding)

- Radiocommunications – communications by means of radio waves

- Signal – physical phenomenon, whose one or several characteristic parameters can express information by its variation

- Simplex – the traffic mode of bidirectional transport of information between 2 points, but not simultaneously

- Routing – the choice (retrieval) of circuits trunk suitable for connecting path building for the actual calling attempt

- Connection (link) – the determinate telecommunications engines and facilities between 2 points

- Session, relation, channel -  temporary routing consisting of transport or telecommunications circuits etc. arranged for allowing information transmission between 2 or more points of TN

- Telecommunication = a) information transport according to conventional rules (standards) by means of follows electromagnetic media as are wires, radio waves, optical media and others,

b) scientific and technical area of communications  electrotechnics...

Reference

[1] T. Anttalainen: Introduction to: Telecommunications Network Engineering, Artech House, 2003, Norwood, USA

[2]  Internet