# Satellite systems and services

Example:    Calculation of antenna diameter at satellite receiving – resume of all relations:

We suppose input specificationsas follows: antenna efficiency 70%, antenna noise temperature 30 K, convertor with NF=1 dB, we are going to receive GEO satellite, frequency bandwidth 27 MHz around 12 GHz. EIRP of satellite is 52 dBW (we have it obtained from properties of satellite). We suppose a „good“ quality of sat. TV picture (it is a little bit history, but the relations are valid, now the quality requirement C/N is more rigorous).

Solution:  (all terms and equations were explained!):

diameter D=?  [m]

receiving antenna gain:  GR=ηπ.Dλ2  [without units].  GR is not yet known.

efficiency η = 70 % , i.e. 0,7

wavelength λ = c/f = 300 . 106/ (12 . 109) = 0,025 m

good quality of receiving Q = 4    C/N = 13 dB   (from table for C/N ).

We know (without units):

CN=Pt.Gt.GR.bp.apk.Tnoise.B  , ....C... power of signal carrier, N ... noise power; N= k. Tnoise.B,

k = 1,38.10-23 WsK-1, Tnoise ... equivalent noise temperature of receiver system; Pt...power of satellite transmitter, Gt ... transmitter antenna gain; Pt . Gt = EIRP,  B=27.106Hz.

GR . . . is not yet known, bp ... propagation loss, ap=other loss (because of atmosphere influences as rain, fog, etc.) = about 0,6 dB.

bp=λ4π.r2... r = 36 .106m ... distance of satellite from Earth, λ=0,025m;

bp = ... = 3,0539 . 10-21 ...   10.log bp = -205,15 dB

Signal-to-Noise Ratio in dB:

[C/N]dB = 10 log (C/N)   = [EIRP + GR + bp + ap – k - Tnoise – B]all in dB =

=  52 dBW + [G/T]ant.;dB – 205,15 dB  – 0,6 dB – (- 228,6dB) – 74,3 dB         (*)

where  kdB = 10 log k = -228,6 dB

adB  = - 0,6 dB

BdB = 10 log B = 74,3 dB.

We can calculate GR/T of antenna with convertor (*):

 [G/T]dB = [C/N]dB  - 52 + 205,15 + 0,6 – 228,6 + 74,3  = 12,45 dB         G/T = 1012,45/10 = 17,58 K-1

G or GR is real gain actually, and  G/T is calculated with losses a, b (a,b – see below):

GT=GR.a.bTa.a+(1−a)T0+(F−1)T0;          T0 ...290 K,

Ta ... noise temper.of antenna ... we suppose for example 30 K

a ...  loss between antenna and convertor (about 0,95)

b ... loss caused of direction and polarization (about 0,9)

F ... noise figure of convertor = 1 dB → without units: antilog(1dB)  =        = 101/10= 1,26 [-]

We resolve GR  from equation for  G/T:

GR =  ...    about 2434    (GR, dBi = 10 log GR =  ... about 34 dBi)

At the end we can calculate D of parabola.

My result is:   D=GRη.λπ=0,47  m